Canada’s River Pulse 2025

Methodology and data sources

Chinmoy Ghosh Shuvo · September 2026

# 1. Purpose

The animation shows every mapped river in Canada on every day of 2025. Each river is drawn as a wall rising from a tilted map: the wall’s height is that day’s discharge and its colour is the estimated water temperature. A counter adds up the discharge of every river mouth on Canada’s coasts (“discharge to the sea”). The design follows Aaron J. Becker’s “South Asia’s River Pulse”.

# 2. Data sources

| Dataset | Used for | Details |
| --- | --- | --- |
| HydroRIVERS v1.0 (North America and Arctic regions) | River network, long-term mean discharge (DIS_AV_CMS), downstream links, river mouths | Lehner & Grill (2013); HydroSHEDS; downloaded as shapefiles from data.hydrosheds.org |
| geoBoundaries CAN ADM0 (gbOpen, simplified) | Canada outline for clipping rivers and drawing the landmass | Runfola et al. (2020); geoBoundaries API |
| GloFAS v4.0 river discharge (0.05°, daily) | Day-to-day behaviour of discharge in 2025 | Copernicus Emergency Management Service; accessed through the Open-Meteo Flood API (Zippenfenig, 2023) |
| NASA POWER daily 2 m air temperature (T2M, MERRA-2 based) | Estimated water temperature (wall colour) | NASA Langley Research Center POWER Project; MERRA-2 (Gelaro et al., 2017) |
| Concept | Visual design idea | Aaron J. Becker, “South Asia’s River Pulse” animation |

# 3. Processing

## 3.1 River network

- HydroRIVERS v1.0 reaches from the North America and Arctic regions were kept if the long-term mean discharge (DIS_AV_CMS) was at least 1 m³/s and the reach midpoint fell inside the geoBoundaries outline of Canada.

- Reaches downstream of Canadian rivers that the outline cut off (estuaries, deltas and the Great Lakes–St. Lawrence chain) were added back when their river mouth lies within 0.05° of Canada. Total: 173,757 reaches.

- River mouths are reaches with NEXT_DOWN = 0 that are not inland sinks (ENDORHEIC = 0) and end within 0.05° of Canada: 3,833 mouths. Rivers that reach the sea through the United States (e.g. Yukon, Columbia, Stikine, Taku) are not counted.

## 3.2 Daily discharge

- 194 sample reaches were chosen: large river mouths first (≥ 200 m³/s, at least 30 km apart), then all reaches ≥ 30 m³/s in order of size, at least 200 km apart.

- Because GloFAS is a 0.05° grid, the nearest cell is often not the river. For each sample, the 3 × 3 neighbouring cells were compared over 1–14 September 2025 and the cell whose mean flow was closest to the HydroRIVERS mean (smallest |ln(Q_cell / DIS_AV_CMS)|) was kept. Samples still more than 3× off were re-screened over a 5 × 5 neighbourhood; those more than 5× off were dropped (17, mostly lakes and deltas), leaving 177.

- Daily 2025 GloFAS discharge was downloaded for each sample. Every reach r then took the pattern of one sample s:

Q(r, t) = DIS_AV_CMS(r) × Q_GloFAS(s, t) ÷ mean_2025[Q_GloFAS(s)]

- The sample used is the first sampled reach downstream (121,710 reaches). If none exists, the largest sample on the same river system (MAIN_RIV) is used (4,131 reaches); otherwise the nearest sample (47,916 reaches).

- Discharge to the sea on each day is the sum of Q over the 3,833 river mouths.

## 3.3 Water temperature (estimated)

- No free daily observed water-temperature product covers Canada, so water temperature was estimated from air temperature.

- NASA POWER daily mean 2 m air temperature was downloaded for 658 points on a 2° grid covering Canada’s rivers (25 Dec 2024 – 31 Dec 2025).

- Each day uses the 7-day trailing mean, interpolated to every reach by inverse-distance weighting (3 nearest points, power 2), and limited to 0 °C or above because river water under ice does not fall below freezing.

## 3.4 Map and animation

- Geometry was projected to NAD83 / Canada Atlas Lambert (EPSG:3978), simplified (400 m tolerance) and compressed vertically by 0.62 to give a tilted view.

- Each wall is a QGIS geometry-generator polygon: extrude(river line, 0, h), with h = 4,000 × √Q metres. Colour runs from teal (0 °C) to orange-red (24 °C and above); larger rivers are more opaque. Walls are drawn north to south so nearer walls cover farther ones.

- Frames (1080 × 1350 px, one per day) were rendered in QGIS 4.2.2 with PyQGIS; titles, the discharge panel, sparkline and legend were drawn with Qt. The 365 frames were encoded with FFmpeg as H.264 (yuv420p) at 24 frames per second, with a 2-second hold on the last frame.

# 4. Results

Discharge to the sea in 2025: low of about 39,100 m³/s on 19 February, peak of about 221,100 m³/s on 29 June, a 5.7-fold swing. The 2025 mean is about 91,500 m³/s.

# 5. Limitations

- Water temperature is estimated from air temperature, not measured.

- Most rivers’ daily flow is estimated: only 177 larger rivers are sampled from GloFAS; every other river borrows the daily pattern of a sampled river, scaled to its own long-term mean.

- The 2025 total is tied to long-term means, so it shows the seasonal rhythm but not whether 2025 was wetter or drier than usual.

- Regulated rivers (e.g. Nelson, La Grande) are only as accurate as GloFAS’s representation of dams and reservoirs.

- GloFAS is a model, not gauge observations.

# 6. References

Copernicus Emergency Management Service. (2019). River discharge and related historical data from the Global Flood Awareness System (Version 4.0) [Data set]. Copernicus Climate Data Store. https://doi.org/10.24381/cds.a4fdd6b9

Gelaro, R., et al. (2017). The Modern-Era Retrospective Analysis for Research and Applications, Version 2 (MERRA-2). Journal of Climate, 30(14), 5419–5454. https://doi.org/10.1175/JCLI-D-16-0758.1

Harrigan, S., Zsoter, E., Alfieri, L., Prudhomme, C., Salamon, P., Wetterhall, F., Barnard, C., Cloke, H., & Pappenberger, F. (2020). GloFAS-ERA5 operational global river discharge reanalysis 1979–present. Earth System Science Data, 12, 2043–2060. https://doi.org/10.5194/essd-12-2043-2020

Lehner, B., & Grill, G. (2013). Global river hydrography and network routing: baseline data and new approaches to study the world’s large river systems. Hydrological Processes, 27(15), 2171–2186. https://doi.org/10.1002/hyp.9740

NASA Langley Research Center. (2026). Prediction Of Worldwide Energy Resources (POWER) Project, daily T2M, accessed 22 September 2026. https://power.larc.nasa.gov. The data was obtained from the NASA Langley Research Center POWER Project funded through the NASA Earth Science Division.

QGIS Development Team. (2026). QGIS Geographic Information System (Version 4.2.2). Open Source Geospatial Foundation. https://qgis.org

Runfola, D., et al. (2020). geoBoundaries: A global database of political administrative boundaries. PLoS ONE, 15(4), e0231866. https://doi.org/10.1371/journal.pone.0231866

Zippenfenig, P. (2023). Open-Meteo.com Weather API [Computer software]. Zenodo. https://doi.org/10.5281/ZENODO.7970649

FFmpeg Developers. (2023). FFmpeg (Version 6.1.1) [Computer software]. https://ffmpeg.org

Becker, A. J. South Asia’s River Pulse [Animated map]. Design concept adapted for this work.